Contamination of groundwater and risk assessment

for arsenic exposure in Ha Nam province, Vietnam

Nguyen V.A., Bang S., Viet P.H., Kim K.-W.

Department of Environmental Science and Engineering, Gwangju Institute of Science and Technology (GIST), South Korea; International Environment Research Center (IERC), Gwangju Institute of Science and Technology (GIST), South Korea; Center of Environmental Technology and Sustainable Development

(CETASD), Hanoi University of Science, Viet Nam

Abstract:  The  characteristics  of  arsenic-contaminated  groundwater  and  the  potential  risks  from  the groundwater were investigated. Arsenic contamination in groundwater was found in four villages (Vinh Tru,

Bo De, Hoa Hau, Nhan Dao) in Ha Nam province in northern Vietnam. Since the groundwater had been used as one of the main drinking water sources in these regions, groundwater and hair samples were collected in the villages. The concentrations of arsenic in the three villages (Vinh Tru, Bo De, Hoa Hau) significantly  exceeded  the  Vietnamese  drinking  water  standard  for  arsenic  (10??g/L)  with  average concentrations of 348, 211, and 325??g/L, respectively. According to the results of the arsenic speciation testing,  the  predominant  arsenic  species  in  the  groundwater  existed  as  arsenite  [As(III)].  Elevated concentrations of iron, manganese, and ammonium were also found in the groundwater. Although more than

90% of the arsenic was removed by sand filtration systems used in this region, arsenic concentrations of most  treated  groundwater  were  still  higher  than  the  drinking  water  standard.  A  significant  positive correlation was found between the arsenic concentrations in the treated groundwater and in female human hair. The risk assessment for arsenic through drinking water pathways shows both potential chronic and carcinogenic risks to the local community. More than 40% of the people consuming treated groundwater are

at chronic risk for arsenic exposure. ?? 2008 Elsevier Ltd. All rights reserved.

Author Keywords: Arsenic; Groundwater; Ha Nam province; Risk assessment; Vietnam

Index Keywords: Arsenic concentrations; Arsenic contaminations; Arsenic exposures; Arsenic speciations; Arsenic species; Average concentrations; Carcinogenic risks; Contaminated groundwaters; Drinking water sources; Drinking water standards; Drinking waters; Elevated concentrations; Ha Nam province; Human hairs; Local communities; Positive correlations; Potential risks; Sand filtrations; Vietnam; Ammonium compounds; Arsenic; Concentration (process); Groundwater; Hydrogeology; Manganese; Nonmetals; Potable water; Risk assessment; Risk management; Underground reservoirs; Water filtration; Water supply; Groundwater pollution; ammonium; arsenic; carcinogen; drinking water; filtration; groundwater pollution; hair; health risk; iron; manganese; pollution exposure; public health; risk assessment; sand; village; Adult; Arsenic; Female; Hair; Humans; Iron; Manganese; Quaternary Ammonium Compounds; Risk Assessment; Rural Population; Soil; Vietnam; Water Pollution, Chemical; Asia; Eurasia; Ha Nam; Southeast Asia; Viet Nam

Year: 2009

Source title: Environment International

Volume: 35

Thank you for trying Solid Converter PDF Professional.

The trial version of this product only converts 10% of your document, with a 10 page maximum.

For this conversion, Solid Converter PDF Professional converted 1 of 4 pages.

Please purchase Solid Converter PDF Professional at http://www.solidpdf.com/buy.htm to remove this restriction.